Introduction:

Learning how to program an LCD Display.

Equipment:

- Simon Board

- Atmel Studio 7

- 2x16 Character LCD Module

- Connection Wires

Procedure:

- Open Atmel Studio 7 and create a new assembly project.

- Properly Wire the LCD Module to Simon board.

- Define the ports and control pins used by the LCD module.

- Start the LCD initialization process by setting PortD and PortE to output mode and clearing the data pins.

- Set the function set to initialize the LCD in 8-bit, 2 line, and 5x7 pixel mode.

- Set the display turn on and cursor on bit to 1 and clear the display bits.

- Write the individual characters of your first name and then move the cursor to the second line.

- Write the individual characters of your last name and clear the display.

- Return the cursor home to begin the loop again.

Results:

The LCD Display spelled out my name on both lines and clear it after it was displayed.

Questions:

- The LCD has three important control pins which are PORTE2, PORTE3, and PORTE4. The pins control read/write, enable, and command/data register. The LCD is initialized to 8-bit mode, 2 line, and 5x7 settings.

- The LCD is level-triggered because the LCD displays the characters by levels.

- I had my name displayed only on the first line. I fixed the issue by setting my last name on the second loop.

Conclusion:

I learned how to program an LCD Display to print out my name and to clear the LCD after the line is displayed.

Appendices:

Appendix 1:

LCD Display

Appendix 2:

Code:

/*

* lab4.c

*

* Created: 3/4/2020 10:45:14 AM

* Author : argk4

*/

#include <avr/io.h>

#define F_CPU 16000000UL

#include "util/delay.h"

#define LCD_DATA PORTD

#define LCD_DATA_DDR DDRD

#define LCD_CNTRL PORTE

#define LCD_CNTRL_DDR DDRE

#define RS PORTE2

#define RW PORTE3

#define EN PORTE4

void lcd_clear();

void lcd_write_data(unsigned char data);

void lcd_write_cmd(unsigned char cmd);

void lcd_ininit();

void lcd_init();

void lcd_return_home();

void move_cursor(unsigned char addr);

int main(void)

{

lcd_ininit();

lcd_init();

while (1)

{

lcd_write_data('A');

lcd_write_data('n');

lcd_write_data('t');

lcd_write_data('h');

lcd_write_data('o');

lcd_write_data('n');

lcd_write_data('y');

move_cursor(0x40);

lcd_write_data('R');

lcd_write_data('o');

lcd_write_data('b');

lcd_write_data('l');

lcd_write_data('e');

lcd_write_data('s');

lcd_clear();

lcd_return_home();

}

}

void lcd_clear()

{

lcd_write_cmd(0x01);

}

void lcd_return_home()

{

lcd_write_cmd(0x02);

}

void move_cursor(unsigned char addr)

{

addr |= (1<<7);

lcd_write_cmd(addr);

}

void lcd_write_data(unsigned char data)

{

LCD_CNTRL |= (1<<RS); // RS = 1, data register

LCD_CNTRL &= ~(1<<RW); // RW = 0, write mode

LCD_DATA = data;

_delay_ms(100);

LCD_CNTRL |= (1<<EN); // EN = 1, enable read or write

_delay_ms(100);

LCD_CNTRL &= ~(1<<EN); // EN = 0, disable read or write

_delay_ms(100);

}

void lcd_write_cmd(unsigned char cmd)

{

LCD_CNTRL &= ~(1<<RS); // RS = 0, command register

LCD_CNTRL &= ~(1<<RW); // RW = 0, write mode

LCD_DATA = cmd;

_delay_ms(100);

LCD_CNTRL |= (1<<EN); // EN = 1, enable read or write

_delay_ms(100);

LCD_CNTRL &= ~(1<<EN); // EN = 0, disable read or write

_delay_ms(100);

}

void lcd_ininit()

{

LCD_DATA_DDR = 0xFF; // port in output mode

LCD_DATA = 0x00; // clear port pins

LCD_CNTRL_DDR |= ((1<<RS)|(1<<RW)|(1<<EN)); //  port pin set to output mode

LCD_CNTRL &= ~((1<<RS)|(1<<RW)|(1<<EN)); // clear port pins

}

void lcd_init()

{

_delay_ms(25);

lcd_write_cmd(0x30);

_delay_ms(10);

lcd_write_cmd(0x30);

_delay_ms(5);

lcd_write_cmd(0x30);

// function set

// 0,0,1,DL=1(8bit), N=1(2line), F= 0(5x8 set), 0,0

lcd_write_cmd(0b00111000);

// display on

// 0,0,0,0,1,D=1(disp on),C=1(Cursor on), B=1(blink)

lcd_write_cmd(0x0F);

// clear display

lcd_write_cmd(0x01);

// Entry mode set

// 0,0,0,0,0,1,I/D=1, SH=0

lcd_write_cmd(0b00000110);

}